Code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity shiftregister is

Port ( clk : in  STD_LOGIC;

reset : in  STD_LOGIC;

datain : in  STD_LOGIC_VECTOR (3 downto 0);

sel : in  STD_LOGIC_VECTOR (1 downto 0);

sl_in : in  STD_LOGIC;

sr_in : in  STD_LOGIC;

dataout : out  STD_LOGIC_VECTOR (3 downto 0));

end shiftregister;

architecture Behavioral of shiftregister is

begin

process(reset, clk, sel, sl_in, sr_in, datain)

variable TEMP : STD_LOGIC_VECTOR (3 downto 0);

begin

if reset = '1' then

TEMP := "0000";

elsif clk'event and clk = '1' then

case Sel is

when "11" => TEMP := datain;

when "01" => TEMP := TEMP(2 downto 0) & sl_in;

when "10" => TEMP := sr_in & TEMP(3 downto 1);

when others =>  NULL;

end case;

end if;

dataout <= TEMP;

end process;

end Behavioral;

RTL Diagram :

Output  :

Left Shift :

Right Shift :

Hardware Output :

Pin Assignment :

| Sr. No. | Signal Name | Pin Designation | Direction | FPGA Pin No. |
| --- | --- | --- | --- | --- |
| 1 | clk | Clock 555: 1Hz | Input | P132 |
| 2 | reset | Key0 | Input | P204 |
| 3 | dataout[0] | TL1 | Output | P205 |
| 4 | dataout[0] | TL2 | Output | P206 |
| 5 | dataout[0] | TL3 | Output | P203 |
| 6 | dataout[0] | TL4 | Output | P200 |
| 7 | datain[0] | TL9 | Input | P192 |
| 8 | datain[1] | TL10 | Input | P193 |
| 9 | datain[2] | TL11 | Input | P189 |
| 10 | datain[3] | TL12 | Input | P190 |
| 11 | SL_in | TL17 | Input | P179 |
| 12 | SR_in | TL18 | Input | P180 |
| 13 | sel[0] | TL25 | Input | P165 |
| 14 | sel[1] | TL26 | Input | P167 |